MMCD Municipal CAD Standard

AutoCAD Civil 3D 2017 Updates

November 2017

Templates General

- The MMCD Municipal CAD Standard now supports Civil 3D versions 2016, 2017 and 2018

- MMCD C3D2017 1.0 TitleBlocks.dwt

  - New layout called ISO A1 Section 100 Scale for 100 scale section view production using C3D plan production tools

AutoCAD Updates

- System Variables

  - Visretain set to 1 (so Xrefs maintain adjusted layer visibility settings)

  - Selectionannodisplay set to 0 (so annotative objects don’t show scale properties when selected)

- Layers

    - Removed R-* (Existing Recorded Assets) from DesignandProduce DWT

    - Reconfigured C3D layers that are referenced by Drawing Settings – Object Layers to be more streamlined and in line with objects they represent

    - Layer C-ROAD-PROF-CNTR now references a Continuous linetype

    - Changed all references from SCTN to SECT

    - Changed HYDR to POWR in all layer names

    - Added the following layers

      - V-PROP-BNDY-MUNI for Municipal boundaries

      - G-ANNO-PNTI for points of interest

      - C-DRAN-FORC for proposed storm sewer force main (no new C3D pipe styles)

      - V-DRAN-FORC for existing storm sewer force main (no new C3D pipe styles)

      - R-DRAN-FORC (Existing Recorded Assets drawing only)

      - V-ROAD-TEXT (Existing Street Names)

      - V-BLDG-BSMT (Under Sidewalk Basements)

      - R-BLDG-BASEMENT (Existing Recorded Assets drawing only)

      - Added R-UTIL* layers and layer group to Existing Recorded Assets drawing

      - Added R-VEGE* layers and layer group to Existing Recorded Assets drawing

    - New collection of existing and proposed pavement marking layers is as follows, which are the same as BC MoTI Civil 3D layering standards:

      - V-ROAD-MRKG (Pavement Marking)

      - V-ROAD-MRKG-ARRW (Direction Arrow)

      - V-ROAD-MRKG-BRKN (Broken White Line, Rural)

      - V-ROAD-MRKG-DECL (Deceleration Lane)

      - V-ROAD-MRKG-URPL (Broken White Line, Urban)

      - V-ROAD-MRKG-WHIT (White Line)

      - V-ROAD-MRKG-XWLK (Cross Walk)

      - V-ROAD-MRKG-YELD (Double Yellow Line)

      - V-ROAD-MRKG-YELO (Yellow Line)

      - V-ROAD-MRKG-YELP (Passing Yellow Line)

      - C-ROAD-MRKG (Pavement Markings)

      - C-ROAD-MRKG-ARRW (Direction Arrow)

      - C-ROAD-MRKG-BRKN (Broken White Line, Rural)

      - C-ROAD-MRKG-DECL (Deceleration Lane)

      - C-ROAD-MRKG-IGLN (Intersection / Bicycle Guiding Line)

      - C-ROAD-MRKG-LNEG (Lane Edge)

      - C-ROAD-MRKG-MEDN (Median)

      - C-ROAD-MRKG-URPL (Broken White Line, Urban)

      - C-ROAD-MRKG-WHIT (White Line)

      - C-ROAD-MRKG-YELO (Yellow Line)

      - C-ROAD-MRKG-YELP (Passing Yellow Line)

      - C-ROAD-MRKG-YELD (Double Yellow Line)

    - Added the following Layer Filter Properties to Existing Recorded Assets drawing

      - Existing Recorded Drainage

      - Existing Recorded Sanitary

      - Existing Recorded Water

      - Existing Recorded Roads

      - Existing Recorded Property

      - Existing Recorded Utilities

      - Existing Recorded Vegetation

- Text Styles

  - Created _Civil 3D Tables text style which is referenced by all C3D Table Styles

  - Created _Civil 3D Profile Views text style which is now referenced by all C3D Profile Views Styles

  - Created _Civil3D Section Views text style which is now referenced by all C3D Section View Styles

  - Modified _Civil 3D Proposed Alignment Stationing to reference Sans Serif font

- Block Definitions

  - See section on Civil 3D Description Keys

- Multileader Styles

  - All multileader style arrow head sizes match the text style

AutoCAD Civil 3D Updates

- AutoCAD Civil 3D

  - Addition of 1_Existing and 2_Proposed Prospector folders for Surfaces and Alignments

  - Label Styles General

    - Reviewed all label styles to correct inconsistencies with AutoCAD text style assignment

  - General Collection (Settings Tab)

    - Cleaned up and consolidated collection of Marker Styles

    - Cleaned up collection of code set styles

  - Points

    - User-Defined Property Classifications

      - Added ATT1, ATT2, ATT3, ATT4 and ATT5 to accommodate attribution fields.  These are found under Survey User-Defined Property Classification category to provide attribute consistency with Survey Database

      - Added DTM (Y or N) for surface inclusion

    - Point Group

      - All Point Groups reference _(x) Cross 0.5 point style

      - EX Surface point group has been renamed to EX Surface (User Defined)

      - Created EX Surface (DTM Attribute) point group that references DTM=Y attribute

      - Renamed _<Off> to All Points (Off)

    - Point Styles

      - Point Styles with text inside the symbol have been updated so the orientation reference has been set to View. This means the that the text will always read left to right regardless of viewport orientation.  The following point styles have been affected:

        - Combined Manhole

        - Sanitary Manhole

        - Water Manhole

      - Added the following Point Styles

        - _Cross (x) 0.5

        - _Circle (o) 0.5

        - _Plus (+) 0.5

    - Point File Format

      - Added PNEZDAttDtm (comma delimited)

    - Label Styles

      - Coordinate colours and made text heights, gaps and offsets consistent across all point label styles

      - Dragged state display properties have all been configured so they are consistent

      - Added Num Elev Full Desc Attributes (Colours) 0.5mm

    - Description Keys and Figure Prefix Database

      - EA now references V-ROAD-EDGE-ASPH layer as opposed to V-ROAD-ASPH

      - Changed CRK to CK so as not to conflict with CR (Crown)

      - SE description key now references the _<off> point styles and point label styles

      - Description Keys referencing blocks with text outside of the block have been updated to reference the Raw Desc 1.8mm point label style.  The corresponding blocks have been modified to remove the text.  This brings more flexibility to base plan creation so that point text can be moved and rotated so it doesn’t conflict with other base plan entities.  Affected point styles include the following:

        - 00 – Misc Spot and Linear Features: ANT, FP, GP, MB, RRS

        - 07 – Utility Hydro: AP

    - Table Styles

      - Added PNEZDAtt

      - Added PNEZDAttDtm

  - Surfaces

    - Modified Command Settings

      - Allow Crossing Breaklines – Yes

      - Elevations to Use – Use average breakline elevation at intersection

    - Surface Styles

      - All surface styles have Section View linetype of Hidden2 with .25 LT Scale

      - Added the following surface styles for viewing in Section only

      - All surface styles now have consistent colours and linetypes for Plan, Model and Section view directions

    - Surface Labels

      - Added EX (Auto Decimal) contour label style and made this the default in Command Settings.  This label style uses an expression to label contours to the decimal precision of the contour

      - Added PR (Auto Decimal) contour label style

  - Alignments

    - Label Styles

      - Modified major station label style Parallel so that dragged state display property does not include a leader

      - Modified major station label style Perpendicular so that dragged state display property displays properly without a leader

      - Modified the alignment label sets to reflect the updated geometry point labeling methodology

      - Reconfigured geometry point label style Tick and Line (Perp) to Geometry Point and Station 025mm Line and reconfigured so text is aligned at the end of the line and the label works better in dragged state.  Added Geometry Point and Station 010mm Line and Geometry Point and Station 050mm Line label styles

      - Fixed station offset label styles for correct text justification

      - Fixed Station ProfileName Elevation station offset label style for improved dragged state display property

    - Offset Alignments and Widenings

      - Adjusted Create Offset Alignment and Add Widening command settings to reference practical values and styles

  - Profile View

    - Profile View Styles

      - Profile view styles have been renamed, reconfigured and appended to.  All grid spacing has been adjusted so ratio of horizontal and vertical grid increments is 1:1. Profile view styles exist for 10H:1V and 5H:1V for 250 and 500 scales.  Profile view elevation text has been resized to 3.5mm.  The complete list of profile view styles is as follows:

  - Profile

    - Label Styles

- Profile label styles have been reconfigured for improved labeling flexibility

- New profile label sets as shown below:

- New profile major station label styles

- New profile grade break label styles

- New profile line label styles

- New profile curve label styles (note that expressions are used to control the tangent in and tangent out angles, based on profile view vertical exaggeration)

  - Corridor

    - Command Settings

      - Region and baseline counters have been reset to 1

      - Note that Create Corridor command settings do not recognized assigned code set style Corridor Design – All Codes and that this code set style needs to be assigned manually after the corridor has been created

    - Marker and link label styles (Reference by Code Set Style ad by Corridor Section Editor)

      - Reconfigured and renamed to be more meaningful and useful

- Pipe Networks

  - Pipes

    - Label Styles

      - Updated the 5 x Name Rim Elevation and Station TOP label styles for proper placement of station and rim elevation.  Also anchored to the MH name label style so distance between this label and the MH name label is consistent, regardless of the length of the MH name

    - Added PR Storm Forcemain (Plan Profile Section) and PR Storm Forcemain Crossing (Profile)

  - Pipe Catalog

    - Circular pipe catalogs have been developed for BC.  These are solely based on materials and are references by the updated Pipe Network Parts Lists in the DWT.  The materials in the pipe parts are referenced be pipe label styles.  The new circular pipe catalogs are as follows:

      - Asbestos Concrete (AC)

      - Brick (BRK)

      - Cast In Place (CIP)

      - Cast Iron (CI)

      - Concrete (CONC)

      - Copper (CU)

      - Corrugated Steel (CS)

      - Ductile Iron (DI)

      - Fused PVC (FPVC)

      - Galvanized Iron (GI)

      - High Density Polyethylene HDPE

      - No Corrode (NC)

      - Not Applicable (NA)

      - PEX-A (PEX)

      - Polyethylene (PE)

      - Polyvinyl Chloride (PVC)

      - PVC Oriented (PVC)

      - Reinforced Concrete (RC)

      - Relined (RL)

      - Stainless Steel (SS)

      - Steel (S)

      - Unknown (UNK)

      - Vitrified Clay (VC)

      - Wood (WD)

  - Updated Parts Lists

    - All new parts lists have been developed based on the updated Pipe Catalog.  These will require modifications based on materials found and used in your jurisdiction.

- Sample Lines

  - C-ROAD-SAMP layer is now colour 12 and linetype (HIDDEN2)

  - Sample line style has been renamed to Sample Line

  - Sample line label style has been renamed to Station and references the C-ROAD-SAMP layer

- Section Views

  - Section view offset labels are now part of the section view style and not in a band

  - Section views have been reconfigured for correct grid spacing and offset / elevation text sizes. The 4 new section view styles include the following:

  - Band styles

    - The default band set is Spacer for Plan Production which has been updated to reference the Spacer for Plan Production section data band style and has been set to the default band set.  This means that the section views and grid labels will not be cut off and will plot properly within the plotted area.

  - Group Plot Style

    - Group plot style Array has been updated to tighten row and column spacing as well as change the order of section view plotting from bottom left to top right

- Section Styles

  - The collection of section styles has been updated and is as follows: